# Chapter 5	Reservoir and Stream-River Network Modeling

5.1	 Overview

The Reservoir Model and the River Model are two major components of WASH123D  a numerical model simulating water and energy budget in reservoirs, water flow and contaminant and sediment transport in watershed systems of one-dimensional stream-river network, two-dimensional overland regime, and three-dimensional subsurface media.

In the work presented in this report, the Reservoir Model and the River Model were used for hydraulic modeling surface water storage areas and canal networks in the study area of northern Palm Beach County. The canal system is composed of the L-8 Canal, the M-Canal, and the East and West Branches of C-18 Canal. The surface water storage areas include a number of reservoirs within the study area.

This chapter describes the mathematical foundation of the modeling work. It also introduces different components (modules) of WASH123D. The chapter is organized as follows. Section 5.2 gives a brief review of some existing hydrologic and hydraulic models. Section 5.3 presents the reservoir module. The river network module is given in Section 5.4. Since the overland module and groundwater module of WASH123D Model are not used in this project, the introductions to these modules are skipped in this chapter. Section 5.5 explains the coupling of reservoir module and river module.

  - Review of Existing Models

The modeling effort in this project can be divided into two parts: 1) hydrologic model; and 2) hydraulic model. The objective of the hydrologic model is to estimate the excess rainfall, and generate runoff input data for the hydraulic model. The hydraulic model is used to study water allocation through canal network system and among surface water storage reservoirs.

5.2.1	Hydrologic Models

In this report, the purpose of the hydrologic model is to provide long-term runoff data for the hydraulic model. There are a number of alternative approaches to develop a hydrologic model. A comparison of different types of hydrologic models are given in Table 5.1

The first type is the parametric model, including the water budget model and the model based on the SCS curve number method. A water budget model provides a long-term simulation of water elevations within surface water conveyance system or other key surface water features as a function of rainfall applied to the study area surface. Water balance is calculated accounting for both inflows and outflows. The inflows consist of rainfall, discharge through inflow boundary and control structures, seepage from groundwater, and supplemental water inflows. Outflows include discharge through outflow boundary and water control structures, evapotranspiration, and seepage to groundwater. The evaporation is calculated using the empirical formulas, such as Penman equation. The seepage is estimated by empirical equations or by output of other groundwater models, such as MODFLOW. A spreadsheet based water budget model for the L-8 Basin, the C-18 Basin, and Grassy Waters Preserve (GWP) (formerly known as West Palm Beach Water Catchment Area) was developed by South Florida Water Management District (SFWMD). (North Palm Beach County Surface Water Model, Contact information: Paul Linton, SFWMD, 2002)

The Soil Conservation Service (SCS) curve number method computes the amount of rainfall runoff, taking abstractions into account, as a function of total rainfall and the curve number (CN), which is an index that represents the combination of hydrologic soil group and land use and treatment class. Empirical analyses suggest that CN is a function of three factors: soil group, the cover complex, and antecedent moisture conditions. The SCS curve number method also assumes that the initial abstraction is a fixed fraction of the maximum retention but varies with soil and land use. Although it was originally intended to model daily runoff as affected by land-use practices, it has been used to model infiltration as well as runoff hydrograph for continuous hydrologic simulation. The SCS method is widely used in many hydrologic models, such as the HEC-1 model developed by Hydrologic Engineering Center of the U.S. Army Corps of Engineers (USACE).

The HEC-1 model is a widely used hydrologic event simulation model. It is designed to simulate the surface runoff response of a river basin to precipitation by representing the basin as an interconnected system of components. Each component models an aspect of the rainfall-runoff process within a sub-basin or sub-area. The components include sub-area surface runoff, stream channels, and reservoirs. The end result of the modeling process is the computation of direct runoff hydrographs for various sub-areas and stream flow hydrographs at desired locations in the watershed. The sub-area surface runoff component is used to represent the movement of water of the land surface and into stream channels. The input to this component is a rainfall hyetograph. Excess rainfall is computed by subtracting infiltration and detention losses, based on an infiltration function that may be chosen from several options, including the SCS curve number loss rate. Rainfall and infiltration are assumed to be uniformly distributed over the sub-basin. The resulting rainfall excesses are then applied to the unit hydrograph to derive the sub-area outlet runoff hydrograph. Alternatively, a kinematic wave model can be used to find sub-basin runoff hydrographs. The HEC-1 model was used to generate the hydrologic data required by a UNET model for the Southern L-8 basin (Southern L-8 Basin Technical Memorandum #3A, SFWMD 1997).

A water balance model solves the continuity equation and a simplified momentum equation for overland flow. Burns & McDonnel has developed a spreadsheet based water balance hydrologic model for the L-8 Basin (2001). The L-8 Basin were partitioned into twenty-two sub-basins. The model estimated stage fluctuations with the sub-basins to account for the effects of inflow and outflow, evapotranspiration, rainfall, soil storage, and seepage. Overland flow was generated by approximating permeable areas, or sub-areas, as non-linear reservoirs. The evaporation was computed by Penman equation. The infiltration capacity was estimated by the Horton’s equation, which is essentially a conceptual model with a semi-empirical formula. For a water balance model, the modeling of infiltration by the Horton’s equation is considered inadequate, an alternative would be the Green-Amp model based on simplified principles of physics.

In WASH123D’s overland flow module, the overland flow is simulated by a distributed model, in which the continuity equation and the momentum equation are solved. In WASH123D, the coupling between overland flow and groundwater is accomplished through the direct coupling of the overland module and subsurface module, where the continuity of the pressure and mass flux are satisfied.

The implementation of the water balance model or the dynamic model requires detailed land surface and subsurface hydrologic data as well as meteorological data. The collection of these data would be an extremely time-consuming process, which is beyond the scope of this project. Thus, based on the availability of required data in this project, a hydrologic water budget model was developed based on lumped information, including the delineation of the L-8 and C-18 Basins, the monthly rainfall and evaporation data on two-mile by two-mile cells covering the study area, and the average SCS curve number provided by SFWMD. The SCS curve number method was used to estimate the excess rainfall. For a long-term simulation, the evaporation was subtracted from the runoff, which was then used as the input data for the River Model.

### Table 5.1 Comparisons of Hydrologic Models

|  | Parametric model |  | Water balance model | Dynamic model |
| --- | --- | --- | --- | --- |
|  | Water budget model | SCS curve number method |  |  |
| Governing equations | Continuity equation | Empirical formula | - Continuity equation of overland flow - Simplified momentum equation, e.g. Manning’s equation - Thermal energy equation (optional) | - Continuity equation of overland flow - Momentum equation of overland flow - Thermal energy equation (optional) |
| Lumped or distributed model | Lumped | Lumped | Lumped/Distributed | Distributed |
| Infiltration | Empirical formula, e.g. Horton’s equation |  | - Empirical formula, e.g. Horton’s equation - Physically based model, e.g. Green-Ampt model. | Physically based model, e.g. Richard’s equation, or Green-Ampt model |
| Evaporation | Empirical formula, e.g. Penman equation |  | Computed by aerodynamic equation when water temperature is part of the solution; or by empirical formula, e.g. Penman equation | Computed by aerodynamic equation when water temperature is part of the solution; or by empirical formula, e.g. Penman equation |
| Required input data | - Rainfall intensity - Land use, soil type, and soil condition | - Rainfall intensity - Land cover/use, soil group, and soil moisture condition; or curve number | - Geometric data of watershed - Land surface hydraulic roughness - Soil property - Rainfall intensity and possibly other meteorological data | - Geometric data of water shed - Land surface hydraulic roughness - Soil property - Rainfall intensity and possibly other meteorological data |
| Coupling with groundwater | Indirect coupling through other models, e.g. MODFLOW | No coupling | Possible indirect coupling through other groundwater models, e.g. MODFLOW | Possible direct coupling with WASH123D’s subsurface module |
| Existing model | SFWMD Water Budget Model for L-8, C-18 Basins and WPB WCA | HEC-1 | Water Balance Model from Burns & McDonnel | WASH123D’s overland flow module |

5.2.2	Hydraulic Models

The comparisons of the hydraulic models for one-dimensional simulation, including the HEC-RAS model, the UNET model, and the WASH123D model, are given in Table 5.2.

The HEC-RAS model is developed by USACE. Currently HEC-RAS is capable of performing one-dimensional water surface profile calculations for steady gradually varied flow in natural or constructed channels. For unsteady flow simulation, HEC-RAS adapts the solver from the UNET model.

The UNET model developed by USACE simulates one-dimensional unsteady flow through a full network of open channels. In this model the continuity equation and the momentum equation are solved by the finite difference method. This model was applied to evaluate the performance of the conveyance facilities by routing three storm events through the L-8 Canal and South L-8 Tie-Back Canal (Southern L-8 Basin Technical Memorandum #3A, SFWMD 1997).

The WASH123D model provides a complete solution package for modeling water and energy budget in reservoirs, water flow and contaminant and sediment transport in watershed systems of one-dimensional stream-river network, two-dimensional overland regime, and three-dimensional subsurface media. The state-of-the-art computational techniques have been integrated into the computer code. The four major modules of WASH123D are the Reservoir Model (0-D model), the River Model (1-D model), the Overland Model (2-D model), and the Subsurface Model (3-D model). Each module can be used separately or coupled with the others. In this report, coupling of the 0-D model and 1-D model will be used to simulate the reservoirs and canal network in the North Palm Beach County area (see Chapter 6 for more details).

### Table 5.2 Comparisons of Hydraulic Models

|  | HEC-RAS | UNET | River Model Module of WASH123D |
| --- | --- | --- | --- |
| Governing equations | For steady flow: - 1-D Energy equation - Manning’s equation For unsteady flow: Same as UNET | - 1-D Continuity equation - 1-D Momentum equation | - 1-D Continuity equation - 1-D Momentum equation - 1-D Energy equation - 1-D Transport equations of sediment, salinity, and chemical concentrations - Chemical reaction equations |
| Kinematic wave approach | Not included | Not included | Included |
| Diffusive wave approach | Not included | Not included | Included |
| Dynamic wave approach | Included for unsteady flow | Included | Included In the form of method of characteristic (MOC) |
| Long-term simulation | Impractical | Impractical | Capable |
| Storm event simulation | Capable | Capable | Capable |
| Numerical method | Direct step method Finite difference method | Finite difference method | Particle tracking and/or finite element method |
| Sediment transport | Capable | Incapable | Capable |
| Simulate water temperature | Incapable | Incapable | Capable |
| Hydraulic structures included | Bridges, culverts, weirs, spillways. | Bridges, culverts, weirs, spillways. | Culverts, weirs, gates, spillways. |
| Continuity equation at junctions | Not satisfied | Not satisfied | Satisfied |
| Simulation reservoir/lake | Linear storage-stage relationship only | Linear storage-stage relationship only | Arbitrary storage-stage relationship |
| Simulate overland flow | Incapable | Incapable | Capable |
| Simulate subsurface flow | Incapable | Incapable | Capable |
| Coupling with overland flow | Indirect | Indirect | Direct through the coupling of the 1-D and 2-D modules. |
| Coupling with subsurface flow | Indirect coupling through other models, e.g. MODFLOW | Indirect coupling through other models, e.g. MODFLOW | Direct through the coupling of the 1-D and 3-D modules. |

  - Reservoir Modeling (0-D Model)

The Reservoir Model includes the flow module and the water temperature module as shown in Table 5.3. As it has been presented in Chapter 2, the flow module solves the water budget equation, while the temperature module solves the energy equation for water body.

### Table 5.3 Components of Reservoir Model

| Module Name | Description | Primitive Variables |
| --- | --- | --- |
| FLOW0D | Represents water budget in the reservoirs (zero-dimensional flow) | Water Stage |
| TEMP0D | Represents energy balance in the reservoirs | Water Temperature |

  - River Modeling (1-D Model)

The Stream-River Model is designed to perform simulation of one-dimensional steady and unsteady flow through a full network of open channels and flow transfer through hydraulic structures, such as weir, gate, and culvert. Showing by Table 5.4, there are three approaches to solve the flow in the River Model: kinematic wave approach, diffusive wave approach, and dynamic wave approach. The one-dimensional transport of thermal energy, salinity, sediment and contaminate are also included in the 1-D Model.

### Table 5.4 Components of River Model

| Module Name | Description | Primitive Variables |
| --- | --- | --- |
| FLOWKIW1D | Represents one-dimensional flow in the river/stream network using the kinematic wave approach | Water Stage |
| FLOWDFW1D | Represents one-dimensional flow in the river/stream network using the diffusive wave approach | Water Stage |
| FLOWDYW1D | Represents one-dimensional flow in the river/stream network using the dynamic wave approach | Water Stage and Velocity |
| TEMP1D | Represents one-dimensional transport of thermal energy in the river/stream network | Water Temperature |
| SALI1D | Represents one-dimensional transport of salinity in the river/stream network | Salinity |

First, we will give the general governing equations for flows in the River Model. Then, the equations for the kinematic wave approach, the diffusive wave approach, and the dynamic wave approaches will be presented in the following sub-sections. Then the transport equation for thermal energy and salinity are given before the end of this section.

The general governing equations for fluid flows in the River Model are derived based on the following assumptions:

- The flow is one-dimensional, i.e., the water stage and velocity vary only in the longitudinal direction of the river.

- The hydrostatic pressure prevails and vertical acceleration can be neglected;

- The bottom slope of the river is small and the bed is fixed;

- The resistance coefficients for steady uniform turbulent flow are applicable so that relationships such as Manning’s equation can be used to describe resistance effects.

- The fluid is incompressible and constant density throughout the flow.

The continuity equation reads

∂A∂ t+∂Q∂ x=S						(5.1)

where t = time; x= the coordinate along the longitudinal direction of the river;A= cross-sectional area; Q = volumetric flow rate of the river; S, the source term, can be expressed as

S=SS+SR−SE−SI+S1+S2

and SS= human-induced source; SR = the source due to rainfall; SE = the sink due to evaporation; SI = the sink due to infiltration; S1 and S2 are the source terms contributed from overland flow.

The momentum equation is given by

∂Q∂t+∂VQ∂x=−gA∂Z0+h∂x−∂Fx∂x+M+Bτs−Pτbρ		(5.2)

where V= cross-sectional mean velocity; h= water depth; Z0 = river bed bottom elevation; Fx = the momentum flux due to turbulence eddy viscosity; ρ = water density; B= the top width of the cross-section. P= the wetted perimeter; τs= surface shear stress; τb = bottom shear stress. Mis the momentum-impulse term, which can be written as

M=MS+MR−ME−MI+M1+M2

where MS = external momentum-impulse from human-induced source/sinks; MR = the momentum-impulse gained from rainfall; ME = the momentum-impulse lost to evaporation; MI = the momentum-impulse lost to the subsurface due to infiltration; M1 and M2 are momentum-impulse gained from overland flow.

Equations (5.1) and (5.2) and properly imposed boundary conditions and initial conditions form a complete system whose solutions describe the flow in one-dimensional channel. Three different approaches may derive from the equation system, including the kinematic wave approach, the diffusive wave approach, and the dynamic wave approach, as will be shown in subsequent subsections.

5.4.1	Kinematic Wave Approach

The kinematic wave model is defined by the following equations.

The equation of velocity:

V=−anR1+∂Z0∂ x2231∂Z0∂ x∂Z0∂ x				(5.3)

where n is Manning’s roughness, ais a unit-dependent factor to make Manning’s roughness unit-independent. R is hydraulic radius. Z0is river bed elevation.

The continuity equation:

∂A∂ t+V∂A∂ x+KA=S						(5.4)

or

DAD t+KA=S						(5.5)

where K=∂V∂ x. DD t=∂∂ t+V∂∂ x denotes the total derivative. Obviously, Equation (5.4) is derived directly from Equation (5.1) and Q=VA.

In WASH123D, Equation (5.5) is solved with the semi-Langrangian method (backward particle tracking) (Yeh, et. al., 1998)

5.4.2	Diffusive Wave Approach

In diffusive wave approach, the momentum equation is simplified and reduced to following equation of velocity:

V=−anR1+∂Z0∂ x2231∂H∂ x∂H∂ x				(5.6)

where n is Manning’s roughness, ais a unit-dependent factor to make Manning’s roughness unit-independent. R is hydraulic radius. His water stage. Z0is river bed elevation.

The continuity equation (5.1) can be rearranged by using the definition of flow rate Q=VA, and substituting equation (5.6) for V, to give

B∂H∂ t−∂∂xK∂H∂ x=S					(5.7)

in which B is the top width of the cross-section, and

K=aAR23n11+∂Z0∂ x2231∂H∂ x				(5.8)

Equation (5.7) is solved using the Galerkin finite element method.

    - Dynamic Wave Approach

For a non-prismatic river, the cross-sectional area can be expressed as a function of the water depth and the coordinate of the longitudinal direction along the river,

A(x,t)=A(h,x)					(5.9)

Differentiating Equation (5.9) with respect to xand t, respectively, gives

∂A∂ t =∂A∂ h ∂h∂t+∂A∂ x ∂x∂t=∂A∂ h ∂h∂t=B∂h∂t				(5.10)

∂A∂ x =∂A∂ h ∂h∂x+∂A∂ x ∂x∂x=∂A∂ h ∂h∂x+∂A∂ x =B∂h∂x+∂A∂ x 		(5.11)

Using Equation (5.10) and (5.11), the continuity equation (5.1) is rewritten as

B∂h∂ t+VB∂h∂ x+A∂V∂ x=S−V∂A∂ x

Dividing both sides of the above equation by B, one has

∂h∂ t+V∂h∂ x+AB∂V∂ x=1BS−VB∂A∂ x				(5.12)

The momentum equation (5.2) can be written as

A∂V∂t+V∂A∂t+∂Q∂x+Q∂V∂x=−gA∂(Z0+h)∂x−∂Fx∂x+M+Bτs−Pτbρ		(5.13)

Substituting the continuity Equation (5.1) into Equation (5.13) and dividing both sides by A yields

∂V∂t+V∂V∂x+g∂h∂x=−g∂Z0∂x−1A∂Fx∂x+1AM−VS+Bτs−Pτbρ		(5.14)

Then, equations (5.12) and (5.14) can be written in matrix form as

∂ E∂ t+A∂ E∂ x=R+D					(5.15)

where

E=h,    A=V AB,   R=R1,   and  D=0

in which

R1=1BS−VB∂A∂x

R2=−g∂Z0∂x+1A−VS+M+Bτs−Pτbρ

D=−1A∂F∂x=1A∂∂xAε∂V∂x

Let c=gAB be the wave speed. It may be shown that the eigenvalues of matrix A are

λ1=V+c

λ2=V−c

The right eigenvectors and the right eigenmatrix of matrix A are

e1=c2g,  e2=−c2g,   L= c2g−c2g1212

and the right eigenvectors and the left eigenmatrix of matrix A are

e1=gc,     e2=−gc,    L−1=gc1−gc1

Hence, the matrix A can be diagonalized as

A=L Λ L−1							(5.16)

where Λ=V+c00V−c is a diagonal matrix.

Substituting Equation (5.16) into Equation (5.15) gives

∂ E∂ t+L Λ L−1∂ E∂ x=R+D					(5.17)

Multiplying both sides of Equation (5.17) by L−1:

L−1∂E∂t+ΛL−1∂E∂x=L−1R+L−1D				(5.18)

Let

δW=L−1δE						(5.19)

Hence it follows from Equation (5.18) and Equation (5.19) that

∂ W∂t+Λ∂ W∂x=L−1R+L−1D					(5.20)

From Equation (5.19), one has

δW=gc1−gc1  δhδV = δV+gcδhδV−gcδh

Let

δω=gcδh

thus ω= gc ds, and

δW= δ (V+ω)δ (V−ω) 						(5.21)

Substituting Equation (5.21) into Equation (5.20), one gets

∂  V+ω∂ t+V+c∂ V+ω∂ x=gcR1+R2+D			(5.22)

∂  V−ω∂ t+V−c∂  V−ω∂ x= −gcR1+R2+D			(5.23)

Designating the total derivative as

DV+cDτ=∂∂t+V+c∂∂x

and   		DV−cDτ=∂∂t+V−c∂∂x,

it gives the following characteristic equations from Equations (5.22) and (5.23)

DV+cV+ωD τ=gcR1+R2+D					(5.24)

DV−c V−ωD τ=−gcR1+R2+D					(5.25)

where the subscripts V+cand V−c refer to the positive and negative characteristics, respectively. Equations (5.24) and (5.25) are solved with the Lagrangian-Eulerian method.

5.4.4	Transport of Thermal Energy

If water temperature is a part of the solution, the following temperature equation based on the principle of conservation of thermal energy should be solved:

∂ (ρ0 Cw A T)∂t+∂(ρ0 Cw VA T)∂x−∂∂xκA∂ T∂x=Φ				(5.26)

where ρ0 = density of water; Cw =  specific heat of water;A = cross-sectional area; T= water temperature; κ= longitudinal heat conductivity coefficient; Φ, the source term, can be expressed as

Φ= ΦS+ΦR−ΦE− ΦI+ Φ1+Φ2+ ΦNet−ΦC

where ΦS = the thermal energy from human-induced source/sinks; ΦR = the thermal energy entering the river due to rainfall; ΦE = the thermal energy utilized for evaporation; ΦI = the thermal energy lost to the subsurface due to infiltration; Φ1 and Φ2 are the thermal energy gained from overland flow.

The transport equation of thermal energy (5.26) is solved by finite element method with properly imposed initial and boundary conditions.

5.4.5	Transport of Salinity

The salinity equation is given as

∂ (A S)∂t+∂( VA S)∂x−∂∂xDSA∂ S∂x=Ψ					(5.27)

where A = cross-sectional area; S= salinity; DS= longitudinal salinity diffusion; Ψ, the source term, can be expressed as

Ψ= ΨS+ΨR+ Ψ1+Ψ2−ΨI

where ΨS = the salinity from human-induced source/sinks; ΨR = the salinity entering the river due to rainfall; ΨI = the salinity lost to the subsurface due to infiltration; Ψ1 and Ψ2 are the salinity gained from overland flow.

The transport equation of salinity (5.27) is solved by finite element method with properly imposed initial and boundary conditions.

5.4.6	Internal and External Boundary Conditions

In the River Model, the one-dimensional river channel is divided into reaches. A reach is a river channel segment bounded by hydraulic structures, river junctions, pump stations, or external boundaries of the modeling area. If the end node of a reach is an internal hydraulic structure or a river junction, the internal boundary conditions summarized in Table 5.5 will be imposed. For internal hydraulic structures, the discharge is obtained from analytic formula or rating curve of the structure. Two types of river junctions are listed in Table 5.5. For a junction without storage capacity, the sum of discharge from all reaches connected at the junction should be zero. For a junction with storage capacity, such as a lake or a reservoir, the end node of the reach will be treated as an external boundary node.

### Table 5.5  Internal Boundary Conditions

| Internal Boundary | Description | Boundary Conditions |
| --- | --- | --- |
| Weir | Represents one-dimensional flow transfer by weirs. | Discharge is determined by weir formula or rating curve of the weir. |
| Gate | Represents one-dimensional flow transfer by gates | Discharge is determined by gate formula or rating curve of the gate. |
| Culvert | Represents one-dimensional flow transfer by culverts | Discharge is determined by culvert formula or rating curve of the culvert. |
| Non-Storage Junction | Represents non-storage junctions of one-dimensional river branches. | Sum of discharge from all reaches at the junction equals to zero. |

If the end node of a reach is an external boundary, an external boundary condition is applied. There are six types of external boundary conditions showing in Table 5.6. The Dirichlet boundary condition gives the water depth or stage as a function of time in the simulation. The discharge at the external boundary can be given as a function of time or in the form of a general rating curve. Two special external boundary conditions are designed to simulate the elevation controlled gate and the demand controlled gate, where the gate openings and the rate at which the gate opens and closes are determine by water elevation and demanding discharge, respectively. On a river node where the river is connected directly to a reservoir or lake, the reservoir/lake boundary condition is imposed. Under this circumstance, the River Model will be solved in coupling to the Reservoir Model.

### Table 5.6  External Boundary Conditions

| Boundary Type | Description | Boundary Conditions |
| --- | --- | --- |
| Dirichlet | Water depth or stage is given at all time. | h = hB(t) |
| Normal Flux | The volumetric flow rate is given at all time. | Q = QB(t) |
| General Rating Curve | The volumetric flow rate is given as a function of water depth or stage. | Q=QB(h) |
| Rating Curve of Elevation Controlled Gate | The volumetric flow rate is given as a function of water elevation and elevation controlled gate opening. | Q=QB(h, Go(h)) |
| Rating Curve of Demand Controlled Gate | The volumetric flow rate depends on water elevation and demand controlled gate opening. The gate opening is given as a function water demanding discharge through the gate. | Q=QB(h, Go(QD)) |
| Reservoir/ Lake | The river is connected to a lake/reservoir. It is used to couple the river flow with on-line reservoirs. | H=HR |

  - River-Reservoir Coupling

The River Model and the Reservoir Model are two major modules of WASH123D used in this project. The water transferred between these modules is modeled by coupling of the 1-D model and the 0-D model. Two types of coupling between the River Model (1-D model) and the Reservoir Model (0-D model), the on-line coupling and the off-line coupling, can be identified and described in subsequent subsections.

5.5.1	On-line Coupling

An on-line reservoir is defined as a reservoir that directly connects to river reaches as shown in Figure 5.1. In the River Model, the coupling is through the external boundary conditions for river nodes at the connection location, where water stage obtained from the Reservoir Model is imposed. In the Reservoir Model, the discharges obtained from the River Model at the connection location are used as inflow and outflow to update the water stage of reservoir. The coupling between the river and the on-line reservoir is modeled in the river-lake module.

Reservoir/

Lake

Stream inflow

Stream outflow

## Figure 5.1  Schematic diagram of an on-line reservoir/lake

5.5.2	Off-line Coupling

An off-line reservoir is defined as a reservoir that does not directly connect to rivers. A simple illustration of the off-line reservoir is shown in Figure 5.2. For an off-line reservoir, the water transfer between the reservoir and the river is accomplished through pump stations and/or hydraulic structures, which are implemented through two auxiliary modules: the pump module, and the gravity-driven hydraulic structure module. A brief introduction to these modules can be found in the subsequent sub-sections.

Reservoir/

Lake

Pump

Structure

## Figure 5.2  Schematic diagram of an off-line reservoir/lake

5.5.3	Gravity-Driven Structures

Conceptually, a gravity-driven structure links two locations and conveys water from one location to another.  These two locations are denoted as site I and site II of the structure. In the coupling of river networks and reservoirs, there are four possible connections for a gravity-driven structure as shown in Table 5.7.

#### Table 5.7 Connection Sites for Gravity-Driven Structure

|  | Site I | Site II |
| --- | --- | --- |
| 1 | Reservoir | Reservoir |
| 2 | Reservoir | River |
| 3 | River | Reservoir |
| 4 | River | River |

In the gravity-driven structure module, three types of hydraulic structures can be used as the connections: weir, gate, and culvert. The discharge rates through the structure are determined from the known water stages at both sites using the analytic formula or rating curves.

5.5.4	Pump Station

Conceptually, the transfer of water form one location to any other location can be accomplished through pump. These two locations are denoted as site I and site II of the pump. Furthermore, the operation of a pump defined by operating rules may depend on water stage or flow of other locations, which are designated as the control site of the pump. The operating rules are predefined and assigned to each pump in the pump module. Examples of pump operating rules can be found in Chapter 7. When river network is coupled with reservoirs, there are four possible connections for a pump station as shown in Table 5.8

#### Table 5.8 Connection Sites for Pump Station

|  | Site I | Site II |
| --- | --- | --- |
| 1 | Reservoir | Reservoir |
| 2 | Reservoir | River |
| 3 | River | Reservoir |
| 4 | River | River |

5.5.5	River-Reservoir Coupling Model

Figure 5.3 shows the structure of combination of the Reservoir Model and the River Model. The input of the model includes the meteorological data, the hydrologic data, the geometric data, definitions of initial and boundary conditions, and facilities information. The required meteorological data have been introduced in Chapter 2. The hydrological data input are generated by the hydrologic model described in Section 5.2.1. It is based on drainage basin information and rainfall. For one-dimensional river network, the geometric data consist of the reach lengths, cross section data, and river junction information. Hydraulic facility data give the locations, dimensions, and connection information of hydraulic structures, such as weirs, gates, culverts, etc. Pump station information are also considered hydraulic facility data. The initial and boundary conditions are also passed to the pre-processing module, which generates input cards for the main program.

The main program is composed of the Reservoir Model and the River Model. The Reservoir Model consists of a water budget module and a thermal energy budget module. The River Model performs simulation of flow, sediment transport, contaminant transport and chemical reaction in one-dimensional river/stream networks. A number of facilities, such as weir, gates, culverts are incorporated into the 1-D model.

The linkage module is designed to simulated connection between different reservoirs and between different locations on the river reach. Two types of facilities are modeled: (1) pump stations; (2) gravity-driven hydraulic structures. Different operating rule may assign to the pump stations and the controlled structures.

The post-processing module generates output files in desired format for data analysis. A set of temporal varying data may be saved for producing special animations as the one on the attached CD-R disk.

Linkage

Hydrological

Data

Preprocessing

Input

Files

Reservoir Model

Water

Budget

Energy

Budget

Pump

Station

Hydraulic

Structure

Post-processing

Output

Data

Animation

Data

Meteorological

Data

Geometric

Data

Initial

Condition

Boundary

Condition

Hydraulic

Facility

Stream-River Model

Flow

Kinematic

Wave

Diffusive

Wave

Dynamic

Wave

Transport

Chemical

Reaction

Temperature

Salinity

Sediment

Chemical

Contaminate

River-Reservoir Coupling Model

River-Lake

Connection

## Figure 5.3 Schematic flowchart of reservoir-river model

Reference

- McCuen, R. H.  1989.  Hydrologic Analysis and Design, Prentice-Hall, Englewood Cliffs, NJ.

- South Florida Water Management District, 1997. Technical Memorandum #2B, Hydrologic Model for the Loxahatchee Slough Basin.

- South Florida Water Management District, 1997. Technical Memorandum #3B, Hydrologic Model for the Loxahatchee Slough Basin.

- South Florida Water Management District, 1996. Technical Memorandum #2A, Hydrologic Model for the Southern L-8 Basin.

- South Florida Water Management District, 1997. Technical Memorandum #3A, Hydraulic Model for the Southern L-8 Basin.

- South Florida Water Management District, 2002. North Palm Beach County Surface Water Model (Spreadsheet in Microsoft Excel format).

- U.S. Army Corps of Engineers Jacksonville District, 2001. Existing Conditions Hydrology and Hydraulic, L-8 Basin, Palm Beach County, Florida. (Contract No. DACW17-99-D-0050, Burns & McDonnel)

- Yeh, G.T., Cheng, H.P., Cheng, J.R, and Lin, H.C., and Martin, W.D.  1998. A numerical model simulating water flow and contaminant and sediment transport in watershed systems of 1-D stream-river network, 2-D overland regime, and 3-D subsurface media (WASH123D: Version 1.0). Technical Report CHL-98-15, U.S. Army Corps of Engineers, Vicksburg, MS. Prepared for Athens Ecosystems Research Division, Office of Research and Development, U.S. Environmental Protection Agency.